NAME 		ZAID BIN ARIF

REG NO 		SP23-BCS-128

PDC 			ASSIGNMENT 02

SUB TO		Prof. AKHZAR NAZIR

# Conceptual Question

Why does choosing a block size that is not a multiple of 32 (warp size) lead to underutilization of GPU hardware resources?

On NVIDIA GPUs, threads run in groups of 32, called a warp.
Think of it like a bus with 32 seats.

- If you launch 40 threads, the GPU needs 2 buses (2 warps = 64 seats).

- But only 40 people get on → 24 seats stay empty.

- Those empty seats are wasted, but the GPU still drives the buses.

So, whenever your block size isn’t a multiple of 32, you waste some GPU power because warps are partly empty.

Explain how occupancy of an SM (Streaming Multiprocessor) depends on block size and threads per block.

Each SM is like a big parking lot where many buses (warps) can park.

- If your block size is chosen well (e.g., 256 threads = 8 full warps), the lot fills efficiently.

- If your block size is weird (e.g., 250 threads = 7.8 warps → rounded up to 8 warps but with empty seats), you waste space.

- This reduces occupancy, meaning fewer useful warps fit in the SM at once, so performance drops.

# Coding Question

import cupy as cp

import numpy as np

import time

# ==============================

# CUDA KERNEL (CuPy RawKernel)

# ==============================

kernel_code = r'''

extern "C" __global__

void invert_image(const unsigned char* input, unsigned char* output, int rows, int cols) {

int x = blockIdx.x * blockDim.x + threadIdx.x;

int y = blockIdx.y * blockDim.y + threadIdx.y;

if (x < cols && y < rows) {

int idx = y * cols + x;

output[idx] = 255 - input[idx];

}

}

'''

invert_kernel = cp.RawKernel(kernel_code, "invert_image")

# ==============================

# HELPER FUNCTION

# ==============================

def run_inversion(img, block_size):

rows, cols = img.shape

d_input = cp.array(img)

d_output = cp.empty_like(d_input)

# Grid dimensions

grid_x = (cols + block_size[0] - 1) // block_size[0]

grid_y = (rows + block_size[1] - 1) // block_size[1]

# Timing

start = time.time()

invert_kernel(

(grid_x, grid_y), block_size,

(d_input, d_output, np.int32(rows), np.int32(cols))

)

cp.cuda.Stream.null.synchronize()

end = time.time()

return d_output.get(), end - start

# ==============================

# MAIN

# ==============================

if __name__ == "__main__":

# Generate test image (1024x1024 grayscale)

img = np.random.randint(0, 256, (1024, 1024), dtype=np.uint8)

block_sizes = [(8, 8), (16, 16), (32, 32)]

results = {}

for block in block_sizes:

_, exec_time = run_inversion(img, block)

results[block] = exec_time

print(f"Block size {block} -> Execution Time: {exec_time:.6f} sec")

# Best config

best_block = min(results, key=results.get)

print("\n Fastest configuration:", best_block, "with time", results[best_block])

# Example output save

best_output, _ = run_inversion(img, best_block)

import cv2

cv2.imwrite("inverted.png", best_output)

print("Inverted image saved as inverted.png")

Output

Block size (8, 8) -> Execution Time: 0.171299 sec

Block size (16, 16) -> Execution Time: 0.000097 sec

Block size (32, 32) -> Execution Time: 0.000131 sec

Fastest configuration: (16, 16) with time 9.72747802734375e-05

Inverted image saved as inverted.png

# Analysis Question

We test 3 cases:

- Case A: 64 threads/block (too small)

- Case B: 256 threads/block (fastest)

- Case C: 1024 threads/block (too big)

👉 Why B is best?

- Case A (64 threads) → Too few threads per block.

  - Many warps (buses) are launched, but SMs are underutilized.

  - The GPU spends more time scheduling tiny blocks instead of doing real work.

- Case B (256 threads) → Sweet spot.

  - Each block has enough threads to keep SM resources busy.

  - Good balance between parallelism and resource usage (registers, shared memory).

- Case C (1024 threads) → Too large.

  - Although it’s the max allowed, each block uses a lot of registers/shared memory.

  - Fewer blocks can run at the same time → less occupancy → slower.

So, neither too small nor too big works best.
The middle value (like 128, 256, 512) often gives optimal performance.

# Discussion Question

Why more threads per block doesn’t always mean faster?

- Register pressure → Each thread needs registers. If too many threads, they run out and spill to slow memory.

- Shared memory limit → A big block may consume all SM shared memory, preventing other blocks from running.

- Scheduling → If blocks are too large, fewer of them fit on an SM → low occupancy → idle GPU cycles.

So, there’s always a balance point (like 128–512 threads per block) where you get the best performance.